Shell-and-Tube Steady Solver

Theoretical Framework, Governing Equations and Empirical Closures

Backbone reference for Helium / LN₂ / Water heat-exchanger results

| Item | Value |
| --- | --- |
| Model | Quasi-1D steady baffled shell-and-tube combustor heat exchanger |
| Configuration | Hot combustion gas inside straight tubes; coolant in shell-side cross-flow |
| Primary implementation | 1Dmodel/main_solve_shellntube.py |
| Wall model | physics/heat_conduction.py — OneDimensionalSteadyConduction_ShellnHelicalTube |
| Shell-side closure | physics/bell_delaware.py (single phase); physics/liquid_flow/ (two-phase, supercritical) |
| Entry point | 1Dmodel/main_steady.py — combustorProp.HX_config = "shellntube" |
| Units | SI throughout, except material property tables which are in °C |

Table 1.  Document scope and source of record. Regenerate this document with docs/generate_shelltube_theory_docx.py after any change to the governing equations or closures.

# 1.  Scope and physical configuration

Hot combustion products from a liquid-fuel/oxygen combustor flow axially inside a bundle of straight tubes. Coolant flows on the shell side, forced into a zig-zag cross-flow path by segmental baffles. Heat passes radially outward from the gas, through the tube wall, into the coolant.

This document specifies the model as implemented, including the empirical correlations and the assumptions attached to each. It is written to be the common technical basis for the Helium, LN₂/supercritical-N₂ and Water results, which differ only in which coolant-side branch of the model is exercised — the gas-side, wall and iteration structure are identical for all three. Fluid-specific consequences are collected in Section 10.

## 1.1  Nomenclature

| Symbol | Quantity | Units |
| --- | --- | --- |
| A_i, A_hot, A_cold | Tube inner flow area; nodal hot- and cold-face areas | m² |
| B | Baffle spacing | m |
| Bo | Boiling number, q″/(G h_fg) | – |
| cp | Specific heat at constant pressure | J/(kg·K) |
| D_i, D_o | Tube inner and outer diameter | m |
| D_s, D_otl | Shell inner diameter; outer tube limit diameter | m |
| dQ_i | Heat rate at node i, per single tube | W |
| f | Darcy friction factor | – |
| G_s, G_w | Shell-side cross-flow and window mass flux | kg/(m²·s) |
| h | Specific enthalpy | J/kg |
| h_g, h_c | Gas- and coolant-side film coefficients | W/(m²·K) |
| h_fg | Latent heat of vaporization | J/kg |
| j | Colburn j-factor | – |
| k_w | Wall thermal conductivity | W/(m·K) |
| L_tube | Tube length | m |
| ṁ_g, ṁ_c, ṁ_tube | Hot-gas, coolant and per-tube gas mass flow | kg/s |
| N, N_tubes, N_b | Axial nodes; tubes in bundle; baffles | – |
| N_tcc, N_tcw | Tube rows crossed in one cross-flow section; in one window | – |
| Nu, Pr, Re | Nusselt, Prandtl, Reynolds numbers | – |
| p, p_r | Pressure; reduced pressure p/p_crit | Pa, – |
| P_t, P_n | Tube pitch; pitch normal to flow | m |
| q″_w | Wall heat flux, outer-area basis | W/m² |
| R_wall | Nodal cylindrical wall thermal resistance | K/W |
| S_m, S_w, S_tb, S_sb, S_b | Bell-Delaware cross-flow, window, tube-baffle, shell-baffle and bypass areas | m² |
| s_w | Tube wall thickness | m |
| T_g, T_c | Bulk gas and coolant temperature | K |
| T_wg, T_wc | Gas-side and coolant-side wall face temperature | K |
| UA | Nodal overall conductance | W/K |
| U_g | Gas velocity | m/s |
| x | Thermodynamic quality (vapour mass fraction) | – |
| X_tt | Lockhart-Martinelli parameter, turbulent-turbulent | – |
| α | Void fraction | – |
| Δx | Axial cell length | m |
| μ, ρ, ν | Viscosity, density, Poisson ratio | Pa·s, kg/m³, – |
| φ | Corrugation severity index, e²/(p D_i) | – |
| ω | Under-relaxation factor | – |

Table 2.  Principal symbols. Subscripts g and c denote the hot gas and the coolant respectively; l and v denote saturated liquid and vapour; b and w denote bulk and wall conditions.

# 2.  Structure of the problem

The boundary conditions determine both the mathematical character of the problem and the solution strategy. Both fluid inlets are prescribed: the gas enters at the tube inlet with a temperature and composition fixed by the upstream combustion solve, and the coolant enters at its own inlet with a prescribed temperature and pressure. Crucially, where those two inlets sit relative to one another depends on the flow configuration.

| Configuration | Gas inlet | Coolant inlet | Mathematical character |
| --- | --- | --- | --- |
| Co-flow | node 0 | node 0 | Initial-value problem. All data is available at one end; a single simultaneous forward march would suffice in principle. |
| Counter-flow | node 0 | node N−1 | Two-point boundary value problem. At node 0 the gas temperature is known but the coolant temperature is not — that is the coolant outlet, the one quantity not prescribed. |

Table 3.  Boundary-condition structure by flow configuration. Counter-flow is what makes an iterative solution necessary; the same solution path is used for co-flow rather than maintaining a second code route.

A two-point boundary value problem cannot be advanced by a single forward march, because half the required data lies at the far end of the domain. The solver therefore decomposes the problem into two one-way marches — a tube-side march and a shell-side march — and iterates them against one another until they are mutually consistent. This is the predictive sweep of Section 8.

WHY THE DECOMPOSITION IS VALID  Each march is a well-posed initial-value problem in isolation. Given a frozen shell-side temperature field, the tube-side march is a pure forward integration; given a frozen duty field, the shell-side energy march likewise. Only the coupling between them is implicit, and it is precisely that coupling which the outer iteration resolves.

A consequence worth recording: because the two marches are already decoupled, changing flow direction is inexpensive. Counter-flow requires only that the shell-side march loop be reversed, together with correct tracking of the coolant's own entrance distance for entrance-length corrections. No shooting method or root-find is required. This differs from the helical-coil solver in the same codebase, which marches wall and coolant together in one coupled pass and must therefore resolve counter-flow by shooting on the hot-end starting enthalpy with adaptive bracketing and bisection.

# 3.  Discretization

A uniform axial grid of N cells spans the tube length, with node i centred at x = (i + ½)Δx:

Δ x = LtubeN    N = 200  (default)	(1)

Node index 0 always denotes the gas-inlet end, irrespective of coolant direction. Co-flow means the coolant also begins at index 0; counter-flow means it begins at index N−1 and marches downward. Maintaining a single index convention for both fluids allows one duty array to serve both marches without reindexing.

Tube-side geometry follows from the outer diameter and wall thickness, and the gas mass flow is divided across the bundle:

Di = Do - 2 sw	(2)

Ai = π Di24

mtube = mgNtubes

PER-TUBE VERSUS BUNDLE QUANTITIES  Every tube-side quantity in this model — velocity, Reynolds number, enthalpy budget and the nodal duty dQ_i — is defined per single representative tube. Multiplication by N_tubes occurs exactly once, where the duty crosses to the shell side (Eq. 26). Using the total gas mass flow in the tube-side velocity is a common and silent error.

S_SB CORRECTED 2026-08-20  The shell-to-baffle leakage area is the diametral gap over the arc where the baffle edge follows the shell: S_sb = Lsb*(Ds/2)*(2*pi - theta_ds) = Ds*Lsb*(pi - theta_ds/2). The implementation previously carried a spurious extra factor of 1/2. Confirmed against the first-principles gap-times-arc derivation and against Hellborg (2017) eq. 47, which agree with each other. Effect on this geometry: S_sb 1.218 -> 2.436 cm2, r_lm 6.51 -> 6.93.

On the shell side the geometry is reduced to the Bell-Delaware area set. The governing area is the minimum cross-flow area at the shell centreline, which sets the reference mass flux for every shell-side correlation:

Sm = B  (Ds - Dotl) + Dotl - DoPn(Pt - Do) 	(3)

# 4.  Tube-side (hot gas) model

A forward march from node 0 to node N−1, evaluated against the current shell-side temperature field.

## 4.1  Enthalpy-removal state parameterization

Rather than carrying temperature as the state variable, the march carries the cumulative specific enthalpy removed from the gas. In the default finite-rate mode the complete thermochemical state is retrieved from a precomputed flamelet/progress-variable (FPV) manifold indexed by two scalars — the removed enthalpy and an unnormalized recombination progress variable Y_c:

(Tg, ρg, μg, kg, cp,g, ωYc) = M(hremoved, Yc)	(4)

Interpolation on the manifold is bilinear. The progress variable is transported by its own net production rate, which is what allows the composition to lag equilibrium as the gas cools; that lag is the finite-rate physics:

dYcdx = ωYc(hremoved, Yc)Ug	(5)

dhremoveddx = 1mtube dQdx

Both are advanced explicitly, node by node. Two comparison modes replace this manifold lookup with a direct chemical-equilibrium solver call at each node, re-equilibrating the mixture at constant pressure after the enthalpy removal (equilibrium mode) or holding composition fixed (frozen mode). Finite-rate is the physically appropriate default for the high heat-extraction regime of interest; the other two are validation modes only.

## 4.2  Dimensionless groups and the gas-side film

Ug = mtubeρg Ai	(6)

Reg = ρg Ug Diμg

Prg = cp,g  μgkg

hg = Nug  kgDi

The Nusselt number and friction factor are dispatched on the tube surface treatment. For grooved (corrugated) tubes, both are taken from the Vicente–Cruz corrugated-tube correlations, parameterized by a corrugation severity index built from groove depth e and groove pitch p:

φ = e2p  Di	(7)

Smooth tubes instead use the standard straight-tube Nusselt dispatch with Colebrook friction. In both cases the result is scaled by calibration multipliers held in the CorrelationCoefficients dataclass, which is the single source of tuning knobs for the model.

## 4.3  Gas-side momentum

dpgdx = - fg ρg Ug22 Di	(8)

FRICTION FACTOR CONVENTION  f_g is the Darcy friction factor throughout the maintained solver paths, consistent with the factor of two in the denominator above. Applying a Fanning-to-Darcy conversion on top of this quadruples the predicted pressure drop.

The coupling here is deliberately one-way: gas pressure is integrated for reporting but is not fed back into the gas density, because the FPV manifold is constructed at a fixed reference pressure. This is admissible because the combustor is designed for gas Mach numbers below approximately 0.3, where the density error incurred by neglecting the pressure drop is small. It is also the reason no quasi-one-dimensional momentum equation is solved for the hot gas anywhere in this model.

# 5.  One-dimensional wall heat-transfer model

At each node the two convective films and the tube wall form a three-resistance series network between the gas and coolant bulk temperatures. The essential geometric feature is that the two films act on different areas, because the tube is cylindrical and the hot fluid is on the inside.

Figure 1.  Radial arrangement and the equivalent nodal resistance network. Because the hot fluid is inside the tube, the gas-side film acts on the smaller inner perimeter and the coolant-side film on the larger outer perimeter.

## 5.1  Geometry and the hot-side mapping

Phot = π Di    Pcold = π Do	(9)

Ahot = Phot  Δ x    Acold = Pcold  Δ x

HOT-SIDE ORIENTATION  The wall model is shared with the helical-coil configuration, in which the hot fluid is outside the tube and the perimeter assignment is reversed. The orientation is selected by a hot_side flag, which must be set to "inner" for shell-and-tube. Setting it incorrectly leaves the network structurally intact but scales both film resistances by D_o/D_i in the wrong direction, producing a plausible but wrong duty.

## 5.2  Wall resistance and overall conductance

The wall resistance is cylindrical, not planar — an important distinction at the wall thickness-to-diameter ratios used here:

Rwall = ln(Di/2 + sw)/(Di/2)2 π Δ x  kw	(10)

The three resistances combine in series to give the nodal conductance and the nodal heat rate:

1UA = 1hg Ahot + Rwall + 1hc Acold	(11)

dQi = UA  Tg,i - Tc,i

## 5.3  Face temperatures

The wall face temperatures follow by walking the resistance chain in order, with the third relation acting as a consistency check that the chain returns to the prescribed coolant temperature:

Twg = Tg - dQhg Ahot	(12)

Twc = Twg - dQ  Rwall

Tccheck = Twc - dQhc Acold   ≡   Tc

## 5.4  Temperature-dependent conductivity and the nodal solve

Wall conductivity is evaluated at the through-thickness mean temperature:

kw = kw(Tw),    Tw = Twg + Twc2	(13)

The wall resistance therefore depends on the very face temperatures being solved for. Each node consequently closes a small nonlinear system, solved to a tolerance of 10⁻⁸ on the residual vector:

F =  Twg - Twgnew,  Twc - Twcnew,  Tccheck - Tccheck,new  = 0	(14)

This nodal nonlinear solve is nested inside the axial node loop, which is in turn nested inside the outer sweep of Section 8 — three levels of iteration in total.

## 5.5  Wall heat flux and radiation

The wall heat flux passed to the shell-side closures is referenced to the outer tube area, since that is the surface the coolant sees:

q''w,i = dQiπ Do Δ x	(15)

A gas radiation model (mean-beam-length gas emissivity, entering as a radiative coefficient in parallel with the convective gas-side film) is implemented in the wall module but is disabled in the shell-and-tube steady path.

# 6.  Shell-side heat transfer and pressure drop

## 6.1  Model altitude

Shell-side flow crosses the tube bundle transversely, turns through a baffle window, and crosses again, alternating along the exchanger. Over most of its path the velocity vector is perpendicular to the axial coordinate, and there is no single streamwise coordinate along which a one-dimensional momentum equation could be posed. The shell side is therefore modelled as zero-dimensional per axial node: a lumped bundle correlation evaluated with local fluid properties. This is not a simplification of a momentum ODE — it is a different and more appropriate model form, empirically calibrated on precisely this geometry.

## 6.2  Bell-Delaware ideal tube bank

Gs = mcSm	(16)

Res = Do Gsμs

Prs = cp μk

The ideal-bank Colburn j-factor uses layout- and Reynolds-dependent piecewise coefficients a₁–a₄ tabulated for triangular, square and rotated-square layouts:

a = a31 + 0.14  Resa4	(17)

j = a1  1.33Pt/Do a Resa2

hideal = j  cp  Gs  Prs-2/3  μbμw 0.14

THE PROPERTY CORRECTION IS EVALUATED, NOT DEFAULTED  The (mu_b/mu_w)^0.14 Sieder-Tate term above is evaluated from the lagged coolant-side wall temperature (Section 8.1). It was previously left at its neutral default of 1.0, i.e. the correlation's own property-variation correction was switched off. Enabling it lowered the water design point's peak wall temperature by 67 K and resolved a convergence stall at that point; it moves the Helium shell-and-tube baseline by about 0.1 %. The ratio is clamped to [0.25, 4] so an unconverged cold-start wall state cannot distort the coefficient.

Five multiplicative correction factors then account for the real bundle's departures from an ideal bank:

hshell = hideal  Jc  Jl  Jb  Js  Jr	(18)

| Factor | Physical effect | Form |
| --- | --- | --- |
| J_c | Baffle-window configuration: tubes within the window experience axial rather than cross flow. F_c is the fraction of tubes in pure cross-flow. | 0.55 + 0.72 F_c |
| J_l | Baffle leakage through tube-to-baffle and shell-to-baffle clearances, which bypasses the bundle. r_lm = (S_sb+S_tb)/S_m, r_s = S_sb/(S_sb+S_tb). | p + (1−p)exp(−2.2 r_lm), p = 0.44(1−r_s) |
| J_b | Bundle bypass around the outside of the tube field. C = 1.35 laminar, 1.25 turbulent; unity when the sealing-strip ratio r_ss ≥ 0.5. | exp(−C F_sbp [1−(2 r_ss)^(1/3)]) |
| J_s | Unequal inlet and outlet end baffle spacing relative to central spacing. | spacing ratio, exponent n₁ = 0.6 |
| J_r | Laminar adverse temperature gradient. Active only below Re ≈ 100, unity above. | (10/N_c)^0.18, blended linearly to 1 at Re = 100 |

Table 4.  Bell-Delaware correction factors. Typical magnitudes for a 25 % baffle cut are J_c ≈ 1.0, J_l ≈ 0.7–0.8, J_b ≈ 0.9, J_s ≈ 1 and J_r = 1 in turbulent flow.

## 6.3  Shell-side pressure drop

Pressure drop is assembled from three zones — the central cross-flow passes, the baffle windows, and the two end zones — each carrying its own leakage and bypass correction (R_l, R_b, R_s):

Δ pideal = 2 f Gs2ρ Ntcc  μbμw -0.14	(19)

Δ pcross = Δ pideal (Nb - 1) Rb Rl

Δ pwindow = (2 + 0.6 Ntcw) Gw22ρ Nb Rl,    Gw = mcSm Sw

Δ pends = Δ pideal  1 + NtcwNtcc  Rb Rs

Δ pshell = Δ pcross + Δ pwindow + Δ pends

### 6.3.1  Two-phase pressure drop

Inside the dome the drop is referenced to the ALL-LIQUID Bell-Delaware value and scaled by the Chisholm two-phase multiplier, rather than evaluating Bell-Delaware at the homogeneous mixture density. The multiplier recovers both limits exactly — unity at x=0 and Gamma-squared at x=1:

Γ = ρlρv  μvμl 0.1	(20)

φ2 = 1 + (Γ2 - 1)  B  x(2-n)/2 (1-x)(2-n)/2 + x2-n

Δ pTP = φ2  Δ pliquid

with n = 0.2 for turbulent flow. B is selected on (Gamma, mass flux); at this exchanger's water design point Gamma = 3.60 and G ~ 3000 kg/m2s, giving B ~ 1.01.

B IS RECONSTRUCTED, NOT TRANSCRIBED  Chisholm (1973) and Grant & Chisholm (1979) were unavailable when this was implemented. The B table is corroborated at exactly one point — Hellborg (2017) eq. 137 hardcodes B = 21/Gamma, which lands precisely in the 9.5 < Gamma < 28 branch. That is encouraging, not verification. Note also that Hellborg's hardcoded branch is WRONG for this exchanger: at Gamma = 3.60 the correct branch gives B ~ 1.01, not 5.83, so copying that equation verbatim would inflate the multiplier roughly sixfold.

### 6.3.2  Momentum (acceleration) pressure drop

As the coolant vaporizes its density collapses and the flow must accelerate, which costs pressure over and above wall friction. Across a cell:

Δ pacc = G2   1ρ i+1 -  1ρ i 	(21)

This is the homogeneous form: the separated-flow momentum term x²/(alpha*rho_v) + (1-x)²/((1-alpha)*rho_l) reduces to 1/rho for the homogeneous void fraction the state closure returns, so a drift-flux alpha would be needed to go further. The term had been omitted entirely; it is worth 26 % of the water design point's total shell-side drop (2.99 of 11.05 bar, where density falls 1000 → 29 kg/m3) and 23 % of the N2 point's.

CORRELATION SET IS A MATCHED PAIR  The ideal-bank j and f coefficient tables and the J/R correction factors were calibrated against one another. Substituting a different tube-bank correlation without re-deriving the corrections invalidates the calibration, even though each component is individually defensible.

## 6.4  Regime dispatch

Bell-Delaware contains no boiling physics whatsoever, so inside the two-phase dome it must be abandoned outright. At supercritical pressure the situation is more subtle: what the property-ratio closures correct for is the steep property variation near the pseudo-critical temperature T_pc(p) — the smeared-out remnant of the latent-heat spike. Away from that region a supercritical fluid is an ordinary single-phase fluid, and the cross-flow-calibrated Bell-Delaware correlation is the better-matched model for a baffled bundle. Selection is therefore made on the local state:

| Local coolant regime | Heat-transfer closure | Friction closure |
| --- | --- | --- |
| Subcooled liquid or superheated vapour (single phase, subcritical) | Bell-Delaware, Eq. 18 | Bell-Delaware, Eq. 19 |
| Saturated two-phase, 0 ≤ x ≤ 1 | Gungor-Winterton, Eq. 22 | Müller-Steinhagen & Heck, Eq. 23 |
| Supercritical, bulk-to-wall interval reaching the pseudo-critical band around T_pc(p) | Property-ratio closure from the registry (McCarthy-Wolf / Taylor) | Registry friction gradient |
| Supercritical, but far from T_pc — no property-ratio correction warranted | Bell-Delaware, Eq. 18 | Registry friction gradient |

Table 5.  Shell-side closure dispatch. Regime is evaluated per node, per sweep, from the running (p, h) state of the coolant and the lagged wall temperature. Note the last row: the heat-transfer closure falls back to Bell-Delaware while the friction model does not — see below.

SUPERCRITICAL PRESSURE ALONE IS NOT THE CRITERION  Selection used to be gated purely on the coolantProp.coolant_model string, so which closure a fluid received was a configuration artefact rather than a statement about its state. Helium and N2 are both supercritical yet sit in entirely different places: Helium at 80 bar has T_pc = 11.4 K against a 300-1400 K march — 26 to 120x above any critical anomaly, with cp flat to 0.1 % — whereas N2 at 88 bar has T_pc = 147.8 K, a bulk of 100-124 K and a wall reaching 164 K, placing the pseudo-critical transition inside the thermal boundary layer. At the N2 design point 97 of 200 nodes meet the test. The selection is latched per node (one-way) so it cannot oscillate while the lagged wall temperature is still rising off its cold seed.

THE PRESSURE MARCH STAYS ON ONE FRICTION MODEL  Bell-Delaware returns a LUMPED whole-bundle pressure drop, apportioned as dp_shell/N, whereas the registry closure returns a LOCAL gradient. Mixing the two across nodes double-counts, because each Bell-Delaware node contributes a share of a whole-bundle drop the other nodes are not taking. A supercritical node that falls back to Bell-Delaware for its heat-transfer coefficient therefore still takes its pressure drop from the local gradient. On this geometry Bell-Delaware's pressure drop is in any case not usable — see Section 11.

## 6.5  Saturated flow boiling

Within the two-phase dome the Gungor-Winterton (1986) correlation is used. Its two-term structure carries the physics: forced convection is enhanced by vapour acceleration, while nucleate boiling is suppressed as that convection thins the near-wall superheated layer.

Rel = G(1-x)Dμl,    hl = 0.023  Rel0.8 Prl0.4 klD	(22)

Xtt =  1-xx 0.9  ρvρl 0.5  μlμv 0.1

Bo = q''G  hfg

E = 1 + 24000  Bo1.16 + 1.37  1Xtt 0.86

S = 11 + 1.15 · 10-6 E2 Rel1.17

hpool = 55  pr0.12 (-log10 pr)-0.55 M-0.5 q''0.67

h = E  hl + S  hpool

The pool-boiling term is the Cooper correlation, with M the molar mass in kg/kmol. The horizontal low-Froude correction of the original reference is not applied; the correlation is used here in its vertical / high-Froude form.

## 6.6  Two-phase friction

The Müller-Steinhagen and Heck correlation interpolates empirically between the all-liquid and all-vapour limits, recovering each correctly at x = 0 and x = 1:

A = fl0 G22 D ρl,    B = fv0 G22 D ρv	(23)

-dpdz =  A + 2(B-A)x  (1-x)1/3 + B x3

## 6.7  Critical heat flux and onset of nucleate boiling

Departure from nucleate boiling is monitored through a lookup-table critical heat flux (Groeneveld 2006), reported as a margin:

CHF margin = q''CHFq''w	(24)

UNDEFINED OUTSIDE THE DOME  The CHF margin is returned as NaN for subcooled liquid, superheated vapour and supercritical states. This is a genuine regime boundary — no critical-heat-flux concept exists there — and not missing data. A margin that falls to zero exactly at x → 1 denotes the dryout transition into complete vaporization, which is the intended behaviour for a design that deliberately superheats its coolant.

Two distinct mechanisms guard the subcooled-to-boiling transition and should not be conflated. The first is a numerical blend window of half-width 0.02 in quality, centred on x = 0, which smooths an otherwise abrupt change in heat-transfer coefficient between the single-phase and two-phase branches. It is a smoothing device with no physical content. The second is the physical criterion, the Bergles-Rohsenow (1964) onset-of-nucleate-boiling wall superheat, evaluated as a diagnostic and reported when exceeded:

q''ONB = 1082  p1.156  1.8  Δ TONB 2.16/p0.0234	(25)

with p in bar, inverted for ΔT_ONB and compared against the estimated wall superheat. Subcooled nucleate boiling can begin well before the bulk fluid reaches saturation if the wall is sufficiently hot; when the solver reports an ONB exceedance it is stating that the smooth numerical blend is standing in for boiling the physics indicates has already begun.

# 7.  Shell-side energy march

The nodal duty computed per representative tube is scaled to the full bundle once, here:

dQtotal,i = dQi  Ntubes	(26)

## 7.1  Single-phase (temperature) march

Ti+1 = Ti + dQtotal,imc  cp,c(Ti)	(27)

## 7.2  Real-fluid (enthalpy) march

hi+1 = hi + dQtotal,imc	(28)

pi+1 = max  pi - Δ pilagged,  1

(T, x, α)i = flashph(pi, hi)

WHY ENTHALPY RATHER THAN TEMPERATURE  A cp·ΔT update has nowhere to place latent heat. Marched in temperature, boiling coolant would continue to rise past 1000 K instead of pinning near saturation while it vaporizes. The deeper distinction is which variable determines the thermodynamic state: for single-phase flow at fixed pressure, h = h(T) is monotonic and invertible, so the two marches carry identical information. Inside the dome, T = T_sat(p) is degenerate — identical at x = 0.1 and x = 0.9 — so temperature cannot represent the state at all, while (p, h) still can.

The single-phase march is a forward-Euler approximation to the enthalpy march, with cp evaluated at each node's starting temperature. It is exact for constant cp and first-order otherwise, with the error scaling as 1/N. For Helium, which is monatomic and has cp flat to roughly 0.1 % between 300 K and 1400 K at 80 bar, the resulting outlet-temperature error is immeasurably small. Section 10 quantifies this per fluid.

## 7.3  Pressure march and lagging

The nodal pressure drop used in Eq. 28 is taken from the previous sweep. This is unavoidable: the pressure profile depends on the enthalpy profile through local density and quality, while the enthalpy profile depends on the pressure profile through the local saturation temperature. Lagging one of the two breaks the circular dependency at first-order accuracy in the sweep iteration, and the lag vanishes at convergence.

## 7.4  Equation-of-state ceiling handling

A very low coolant flow against a large duty can superheat the coolant past the property library's validity ceiling. This is a genuine physical edge case rather than a numerical failure. When the flash fails, the last valid state, enthalpy and pressure triple is held for the remainder of the march. All three must be frozen together; freezing only the state while enthalpy and pressure continued to drift would re-trigger the same failure on the following sweep, when those drifted values are read back.

# 8.  Outer iteration (predictive sweep)

Figure 2.  The predictive sweep. The two marches exchange a duty field forward and a temperature field back. Three closure inputs cannot be evaluated before the march that produces them and are therefore consumed one sweep behind.

The two marches are closed by an under-relaxed Picard (fixed-point) iteration:

tube ← march(Tshell(k))	(29)

Tnew ← shell march(dQ)

δ = max  Tnew - Tshell(k)

Tshell(k+1) = (1-ω) Tshell(k) + ω  Tnew

Iteration continues until δ < 0.05 K, to a maximum of 25 sweeps. Under-relaxation at ω = 0.5 is functionally necessary rather than cosmetic: the coupling is stiff, since the coolant film coefficient depends on the shell temperature, which depends on the duty, which in turn depends on the film coefficient. An unrelaxed update oscillates or diverges, particularly across the boiling transition where the sensitivity of the film coefficient to shell temperature is greatest.

## 8.1  Lagged closure quantities

| Quantity | Consumed by | Seeded at | Reason for lag |
| --- | --- | --- | --- |
| q″_w | Boiling-number term in Gungor-Winterton | 0 | Wall flux is an output of the tube-side march that the shell-side closure needs as input |
| Δp per node | Shell-side pressure march | 0 | Pressure and enthalpy profiles are mutually dependent |
| T_wc | Property ratio in supercritical closures | coolant inlet T | Coolant-side wall temperature is an output of the wall solve |

Table 6.  Quantities lagged by one sweep iteration. All are seeded neutrally and all converge to self-consistency as δ → 0.

## 8.2  Initialization

SEEDING DIFFERS BY COOLANT MODEL  Single-phase mode can seed the shell field with a linear profile between the two prescribed inlets. Real-fluid mode cannot: a prescribed coolant outlet temperature is meaningless when the coolant may be two-phase, because inside the dome T and p are not independent and a single (T, p) pair cannot represent the state. Real-fluid mode therefore seeds uniformly at the physical subcooled inlet condition.

## 8.3  Convergence behaviour

Convergence is monotone for well-conditioned single-phase cases. Near a boiling transition the iteration may instead settle into a limit cycle of order 0.5–1 K amplitude without driving δ below tolerance. The reported outlet is stable to within that band, but this is a genuine non-convergence and is reported as such rather than being silently accepted. Where a design point exhibits this behaviour it should be recorded alongside the result.

## 8.4  Global energy closure

Qtot = i dQi  Ntubes 	(30)

The gas side is always enthalpy-marched, so the duty subtracted from the gas budget is exactly what crosses the wall. Whether the coolant side closes exactly depends on which march of Section 7 was used:

Real-fluid (enthalpy) mode — exact by construction. Enthalpy is the marched variable, so the coolant absorbs precisely the summed duty by definition. No closure error exists to monitor.

Single-phase (temperature) mode — first order. Temperature is accumulated using cp at each node's start, so the true enthalpy rise and the summed duty differ in general. The discrepancy is zero for constant cp and diminishes as 1/N.

In neither case is there time-integration drift to accumulate, which is why the steady solver carries no energy-residual diagnostic, unlike the transient solvers where such a diagnostic is essential.

# 9.  Structural check

The tube loading in this configuration is the reverse of a regeneratively cooled channel: high-pressure coolant surrounds the tubes while comparatively low-pressure gas flows inside. The tubes are therefore under external pressure and the failure mode of interest is elastic collapse rather than burst. This check is invoked separately from the thermal solve.

Δ Pext = pc,in - p0	(31)

σext = σext(Δ Pext, sw, Di)

Pcr = Pcr(E(T), sw, Di, ν)

collapse margin =  Δ Pext Pcr   < 1   for safety

Young's modulus is evaluated at the mean of the maximum gas-side and minimum coolant-side wall temperatures, converted to degrees Celsius, since the material property tables are tabulated in °C while the solver is otherwise SI throughout.

EXTRAPOLATION BEYOND THE CHARACTERIZED RANGE  Inconel 718 property data spans −240 °C to 760 °C (1033.15 K). Above that limit the yield curve clamps flat rather than extrapolating a physical material law, so a stress ratio computed at, say, 1100 K is arithmetically well-formed but not physically supported. Maximum wall temperature must always be reported alongside the collapse margin so that this condition is visible.

# 10.  Application to Helium, LN₂/N₂ and Water

The three coolants of interest exercise different branches of the model. The gas-side, wall and iteration structure is identical in all three cases; only the shell-side closure and the energy-march variable differ. This section is the bridge between the framework above and the reported results.

| Coolant | Typical regime at design point | Shell-side closure | Energy march | Status |
| --- | --- | --- | --- | --- |
| Helium | Single phase throughout, well above critical pressure but gas-like in behaviour | Bell-Delaware | Eq. 27 | Baseline. cp is flat to ≈ 0.1 % over the duty range, so the temperature march is numerically indistinguishable from an enthalpy march. |
| LN₂ / supercritical N₂ | Supercritical throughout at any pressure able to meet a 75 bar exit target, since p_crit ≈ 34 bar. Quality undefined; liquid-like dense fluid. | Registry property-ratio closure (McCarthy-Wolf / Taylor) | Eq. 28 | Bounded sensitivity. The supercritical correlations remain straight-tube fits applied to cross-flow, but substituting Bell-Delaware moves Q_tot by only 1 % (the coolant film is 0.58 % of total resistance); wall temperature carries ~54 K of uncertainty. |
| Water | Traverses subcooled liquid, the two-phase dome, and superheated vapour within a single exchanger when the design targets a steam outlet. | Bell-Delaware → Gungor-Winterton → Bell-Delaware, with blending at the dome entry | Eq. 28 | Partly validated. The boiling correlations are literature-validated for tube flow; cross-flow enhancement of boiling is real and not modelled. |

Table 7.  Coolant-specific model paths. The common caveat for the two real-fluid cases is that Gungor-Winterton, Müller-Steinhagen-Heck and the supercritical property-ratio closures are all tube-flow correlations applied to shell-side cross-flow.

CROSS-FLOW BOILING ENHANCEMENT  Shell-side cross-flow over a tube bundle enhances boiling heat transfer relative to flow inside a tube, through mechanisms none of the correlations used here represent. The model is therefore expected to be conservative on coolant-side heat-transfer coefficient in the two-phase and supercritical branches. This is a known open item, not a modelling oversight, and it is surfaced by the solver at runtime rather than left silent.

# 11.  Assumptions and validity

| Assumption | Rationale | Where it becomes significant |
| --- | --- | --- |
| Bell-Delaware baffle-leakage ratio r_lm is far outside the correlation's fitted range | r_lm = (S_sb+S_tb)/S_m = 6.5 on this geometry against a fitted range of roughly r_lm <= 1: the tube-to-baffle clearance area (17.7 cm2) is 6.5x the cross-flow area (2.9 cm2) | Both leakage corrections are extrapolated: J_l = 0.41 on heat transfer, and R_l = 0.009 on pressure drop. R_l crushes the cross-flow and window terms, leaving the uncorrected end-zone term to dominate a whole-bundle drop of ~288 bar. Shell-side pressure drop from Bell-Delaware is not usable on this geometry; reported exit pressures come from the local friction-gradient model. Affects every Bell-Delaware result including the Helium baseline. Reported at runtime. |
| Two-phase and supercritical closures are tube-flow correlations applied to cross-flow | No validated shell-side cross-flow boiling or supercritical closure is available | Any two-phase shell-side case. For the supercritical N2 design point the sensitivity has been measured: substituting Bell-Delaware changes h_c by 3x but Q_tot by only 1 %, because the coolant film carries just 0.58 % of the total thermal resistance (gas film 95.2 %, wall 4.2 %). Wall temperature carries ~54 K of closure uncertainty. |
| Shell-side pressure drop apportioned uniformly across axial nodes (single-phase branch) | Bell-Delaware returns a lumped whole-bundle drop; a per-baffle split would require its per-crossing terms to be re-derived | When local saturation temperature matters and the true drop is strongly non-uniform axially |
| Closure quantities lagged by one sweep | Breaks an otherwise circular dependency between the two marches | First order in the sweep; vanishes at convergence but contributes to limit cycling where convergence stalls |
| Gas pressure does not feed back into gas density | The FPV manifold is constructed at a fixed reference pressure | Only if gas Mach number rises materially above the 0.3 design ceiling |
| Shell-side single-phase branch holds pressure at the inlet value | Enthalpy of a near-ideal gas is weakly pressure-dependent | Density and transport properties do vary with pressure; the computed Bell-Delaware drop is presently discarded in this branch |
| Shell side is zero-dimensional per node with no momentum ODE | Baffled cross-flow has no single streamwise coordinate; this is the appropriate model form | Not a limitation in the modelled regime |
| Gas radiation disabled | Model implemented but switched off in this solver path | Very high gas temperature combined with high gas emissivity |
| Ideal-gas relations in the legacy quasi-1D coolant equations | Applies only to the helical-coil solver, not to this one | Not applicable to shell-and-tube |

Table 8.  Model assumptions and their domains of validity, ordered approximately by practical impact on the results reported in this programme.

## 11.1  Self-reporting diagnostics

Several of the limitations above surface at runtime rather than remaining implicit in documentation. When reading a solver run, the following messages are load-bearing and should be recorded with the result:

Supercritical cross-flow geometry extrapolation warning, naming the selected closure.

Bergles-Rohsenow onset-of-nucleate-boiling exceedance, with the wall-superheat margin.

Sweep non-convergence notice, with the final residual.

NaN rather than a fabricated value for CHF margin outside the two-phase dome.

Regimes traversed and count of heat-transfer-deterioration risk nodes in supercritical operation.

Bell-Delaware baffle-leakage ratio r_lm exceeding the correlation's fitted range.

Count of supercritical nodes whose heat-transfer coefficient came from Bell-Delaware rather than a property-ratio closure.